DATE: 7/10/2026

EXP1 Estimation of Characteristic Impedance (Zo), and Propagation Constant (γ) of a Transmission Line from Primary Constants

TEST CASE 1:

AIM

Investigating the significance of Characteristic Impedance (Zo), and Propagation Constant (γ) of a Transmission Line with Resistance =10 Ω/Km, Capacitance = 8 µF/Km, Inductance =3.5 mH/Km, Conductance =4 µmho/Km, Frequency= 2 Khz

SOFTWARE REQUIRED:

SCILAB Version 6.1.1

PROCEDURE:

- Open SCILAB software

- Open file-new

- Put the input values

- Type the program

- Execute the program

- End the program

PROGRAM:

clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

printf('-Phase constant b = %f radians/km',b1);

OUTPUT TABULATION:

| Simulation Results |  |  | Theoretical Results |  |
| --- | --- | --- | --- | --- |
| Z0(ohms) | α(neper/km) | β(radians/km) | α(neper/km) | β(radians/km) |
| 14.000000 + j(-2.000000)ohms | 0.362100 neper/km | 2.776900 radians/km |  |  |

RESULT:

The characteristic impedance and propagation constant of a transmission line is calculated using SCILAB and the impact of primary constants on secondary constants are investigated.

TEST CASE 2:
AIM:

To examine the impact of resistance and inductance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line at a frequency f=2.5 Khz. Vary the resistance value from 10 Ω to 50 Ω at the interval of 10 Ω with L,G and C values constant and calculate the Characteristic Impedance and Propagation Constant . Vary the inductance values from 3.5 mH to 11.5 mH at the interval of 2.5 mH with R,G and C values constant.

SOFTWARE REQUIRED:

SCILAB Version 6.1.1

PROCEDURE:

- Open SCILAB software

- Open file-new

- Put the input values

- Type the program

- Execute the program

- End the program

PROGRAM:

clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

printf('-Phase constant b = %f radians/km',b1);

TABULATION:

| S.No. | Inputs (R) | Simulation Z₀ (Ω) | Simulation α (Np/km) | Simulation β (rad/km) | Theoretical α (Np/km) | Theoretical β (rad/km) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 20 − j3 | 0.2560 | 1.9635 | 0.2560 | 1.9635 |
| 2 | 20 | 20 − j5 | 0.5002 | 2.0100 | 0.5002 | 2.0100 |
| 3 | 30 | 21 − j7 | 0.7258 | 2.0777 | 0.7258 | 2.0777 |
| 4 | 40 | 21 − j9 | 0.9316 | 2.1582 | 0.9316 | 2.1582 |
| 5 | 50 | 22 − j11 | 1.1192 | 2.2456 | 1.1192 | 2.2456 |

OUTPUT:

TEST CASE 3:

AIM:

To explore the impact of frequency and capacitance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line. Vary the frequency from 2KHz to 10 KHz at the interval of 2 KHz with L,R ,G and C values constant and calculate the Characteristic Impedance and Propagation Constant

SOFTWARE REQUIRED:

SCILAB Version 6.1.1

PROCEDURE:

- Open SCILAB software

- Open file-new

- Put the input values

- Type the program

- Execute the program

- End the program

PROGRAM:
clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

printf('-Phase constant b = %f radians/km',b1);

TABULATION:

| S.No. | Inputs (C) F | Simulation Z₀ (Ω) | Simulation α (Np/km) | Simulation β (rad/km) | Theoretical α (Np/km) | Theoretical β (rad/km) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 µF | 34 − j1 | 0.1490 | 4.2175 | 0.1490 | 4.2175 |
| 2 | 10 µF | 30 − j1 | 0.1666 | 4.7153 | 0.1666 | 4.7153 |
| 3 | 12 µF | 27 − j1 | 0.1825 | 5.1654 | 0.1825 | 5.1654 |
| 4 | 14 µF | 25 − j1 | 0.1971 | 5.5793 | 0.1971 | 5.5793 |
| 5 | 16 µF | 24 − j1 | 0.2107 | 5.9645 | 0.2107 | 5.9645 |

OUTPUT:

RESULT:
The impact of frequency and capacitance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line is investigated for various values of frequencies and capacitance values.