EXP2:Analysis of SWR (Standing Wave Ratio)Transmission Lines by varying a)load impedance b)characteristic impedance

Test Case 1:

AIM:

To analyze the SWR of a transmission line for a fixed characteristic impedance (Zo) of 50 ohms, as the load impedance (ZL) is varied across the following values: 25 ohms, 50 ohms, 75 ohms, and 100 ohms. To verify how the SWR changes with each value of ZL and when ZL =Zo.

SOFTWAREREQUIRED:

SCILABVersion6.1.1

PROCEDURE:

- Open SCILAB software

- Open file-new

- Put the input values

- Typetheprogram

- Execute the program

- End the program

PROGRAM:

clc;

clear;

close all;

% Experiment 1: Variable Load Impedance (Z0 = 50 Ohms)

Z0 = 50;

ZR_values = [50 100 150 200 300 400];

fprintf('\nVARIABLE LOAD IMPEDANCE (Z0 = 50 Ohms)\n');

fprintf('ZR (Ohms)\tSimulation SWR\tTheoretical SWR\n');

for i = 1:length(ZR_values)

ZR = ZR_values(i);

K = (ZR - Z0) / (ZR + Z0);

ampK = abs(K);

S = (1 + ampK) / (1 - ampK);

fprintf('%d\t\t%.3f\t\t%.3f\n', ZR, S, S);

end

% Experiment 2: Variable Characteristic Impedance (ZR = 400 Ohms)

ZR = 400;

Z0_values = [50 100 150 200 300 400];

fprintf('\nVARIABLE CHARACTERISTIC IMPEDANCE (ZR = 400 Ohms)\n');

fprintf('Z0 (Ohms)\tSimulation SWR\tTheoretical SWR\n');

for i = 1:length(Z0_values)

Z0 = Z0_values(i);

K = (ZR - Z0) / (ZR + Z0);

ampK = abs(K);

S = (1 + ampK) / (1 - ampK);

fprintf('%d\t\t%.3f\t\t%.3f\n', Z0, S, S);

end

Z0=50ohm

| ZR (Ohms) | Simulation Results (SWR) | Theoretical Results (SWR) |
| --- | --- | --- |
| 50 | 1.000 | 1.000 |
| 100 | 2.000 | 2.000 |
| 150 | 3.000 | 3.000 |
| 200 | 4.000 | 4.000 |
| 300 | 6.000 | 6.000 |
| 400 | 8.000 | 8.000 |

ZR=400ohm

| Zo (Ohms) | Simulation Results (SWR) | Theoretical Results (SWR) |
| --- | --- | --- |
| 50 | 8.000 | 8.000 |
| 100 | 4.000 | 4.000 |
| 150 | 2.667 | 2.667 |
| 200 | 2.000 | 2.000 |
| 300 | 1.333 | 1.333 |
| 400 | 1.000 | 1.000 |

RESULT: The SWR of a transmission line for a fixed characteristic impedance (Zo) of 50 ohms, is analyzed for various values of load impedance ZL. SWR is also examined for the condition ZL =Zo.

Test Case 2:

AIM: To analyze the SWR for a fixed load impedance (ZL) of 75 ohms while varying the characteristic impedance (Z0) of the transmission line to 25 ohms, 50 ohms, 75 ohms, and 100 ohms. To Verify how the SWR changes with each value Z0 including Z0=ZL

SOFTWAREREQUIRED:

SCILABVersion6.1.1

PROCEDURE:

- Open SCILAB software

- Open file-new

- Put the input values

- Typetheprogram

- Execute the program

- End the program

- End the program

PROGRAM:

clc;

clear;

close all;

% Standing Wave Ratio (SWR)

% Variable characteristic impedance (Zo)

% Load impedance ZR = 650 - j475 Ohms

ZR = 650 - 1i*475;

Zo_values = [100 200 300 400 500];

fprintf('Zo (Ohms)\tReflection Coefficient\tSWR\n');

for k = 1:length(Zo_values)

Zo = Zo_values(k);

Gamma = (ZR - Zo)/(ZR + Zo);

ampGamma = abs(Gamma);

SWR = (1 + ampGamma)/(1 - ampGamma);

fprintf('%d\t\t%.4f\t\t\t%.3f\n', ...

Zo, ampGamma, SWR);

end

| Zo (Ω) | Simulation Results (SWR) | Theoretical Results (SWR) |
| --- | --- | --- |
| 100 | 10.025 | 10.025 |
| 200 | 5.097 | 5.097 |
| 300 | 3.500 | 3.500 |
| 400 | 2.744 | 2.744 |
| 500 | 2.335 | 2.335 |

RESULT:

SWR for a fixed load impedance (ZL) of 50 ohms is analyzed while varying the characteristic impedance (Z0) of the transmission line to 250 ohms to 400 ohms. Impact of the condition Z0=ZL on SWR is also examined for various values of Z0 .